Study of Degrees of Freedom, Joints, Links and Mechanisms

Vyas Bodke

Research Subsystem

RoboManipal, MIT

Manipal, India

vyas.mitmpl2026@learner.manipal.edu

Abstract—This report explains links, joints, degrees of freedom, and mechanisms used in robots. Types of joints and their degrees of freedom with examples, along with Gruber’s formula and calculate the degrees of freedom of some examples. Basics of Stewart platform is also done.

Key words—degrees of freedom, joints, links, mechanisms,

Grubler’s formula, Stewart platform

# I. INTRODUCTION

Links, joints and their degrees of freedom are explained here. These are used in robotics.

A link is a rigid part of a mechanism, while a joint connects two links and allows some movement between them. The type of unique movement possible in a system is described using degrees of freedom.

Understanding Dof is important because it tells us how many different movements a mechanism can make. For ex, a door can only rotate about its hinge, so it has one Dof. A robotic arm can have several independent movements and therefore can have several degrees of freedom.

II. LINKS

A link is a rigid part that is connected to other parts. We assume that the link does not change its shape while the system is moving.

For example, the different arms of a robotic arm can be considered as links. A link can be connected to another link using a joint.

## A. Types of Links

Links can be classified in different ways depending on their use and shape.

- Binary Link: A binary link is connected to other links at two points.

- Ternary Link: A ternary link is connected to other links at three points.

- Quaternary Link: A quaternary link is connected to other links at four points.

These names mainly tell us how many joints are present on a link.

## B. Fixed Link

One link in a mechanism is usually treated as fixed. It provides a reference for the movement of the other links. It is also called the ground link or frame.

For example, in a four bar mechanism, one of the four links is fixed while the other three move.

III. JOINTS

A joint connects two links and allows them to move relative to each other. The type of movement allowed by the joint depends on the type of joint.

Joints also remove some possible movements of the connected links. Therefore, joints are very important when calculating the total degrees of freedom of a system.

## A. Revolute Joint

A revolute joint allows rotation around one fixed axis. It is similar to the hinge of a door.

A revolute joint has one degree of freedom.

Dof = 1

It is commonly represented by the letter R.

## B. Prismatic Joint

A prismatic joint allows one link to slide along a straight line. It does not allow rotation between the connected links. It can have rectangular shape.

A prismatic joint also has one degree of freedom and is represented by P.

Dof = 1

## C. Cylindrical Joint

A cylindrical joint allows both rotation and sliding along the same axis.

Therefore, it has two independent degrees of freedom.

DOf = 2

## D. Spherical Joint

A spherical joint allows rotation about three axes. Our shoulder is an example.

It allows three rotational movements about the three axes and has three degrees of freedom.

Dof = 3

## E. Planar Joint

A planar joint allows movement in a plane. It can allow movement in two directions and rotation about the axis perpendicular to the plane.

Therefore, it has three degrees of freedom.

Dof = 3

## F. Helical or Screw Joint

A helical joint allows rotation and translation at the same time, but the two movements are connected to each other. A screw is an example.

Since the translation depends on the rotation, it has one independent degree of freedom.

Dof = 1

# IV. DEGREES OF FREEDOM

Degrees of freedom, or Dof, describe the number of independent movements possible in a system.

A free object in three-dimensional space can move in three directions and rotate about three axes. These are:

- Movement along the X-axis

- Movement along the Y-axis

- Movement along the Z-axis

- Rotation about the X-axis

- Rotation about the Y-axis

- Rotation about the Z-axis

Therefore, a completely free rigid body in three-dimensional space has six degrees of freedom.

Dof = 3+3 = 6

In a two-dimensional plane, a rigid body can move in the X and Y directions and rotate about the axis perpendicular to the plane. Therefore, it has three degrees of freedom.

Dof = 3

## A. Independent Movement

If one movement automatically causes another movement, they are not counted as two independent degrees of freedom. For example, the screw in screw joint rotates and also moves forward. But, the forward movement is dependent to the rotation. Therefore, a screw joint has one independent Dof.

# V. KINEMATIC PAIRS

When two links are connected in such a way that their relative movement is restricted, they form a kinematic pair.

The common types of kinematic pairs used in mechanisms include lower pairs and higher pairs.

## A. Lower Pair

In a lower pair, the two connected parts have surface contact with each other.

Examples include:

- Revolute joint

- Prismatic joint

- Cylindrical joint

- Spherical joint

## B. Higher Pair

In a higher pair, the two parts generally have contact at a point or along a smaller area.

# VI. MECHANISMS

A mechanism is a group of links connected by joints in such a way that a desired motion can be done.

The links and joints work together to control the movement of the mechanism.

# VII. GRUBLER’S FORMULA

Grubler’s formula is used to calculate the degrees of freedom of a planar mechanism.

For a planar mechanism, the formula can be written as:

F = 3(n −1)−2J − H	(1)

where:

- F is the degrees of freedom of the mechanism.

- n is the total number of links, including the fixed link.

- J is the number of lower-pair joints.

- H is the number of higher-pair joints.

The formula is useful because it gives a quick way to find the number of independent movements possible in a planar mechanism.

## A. Why Three is Used

In a plane, a free link has three possible movements:

- Translation along X

- Translation along Y

- Rotation

Therefore, each free link starts with three possible degrees of freedom.

One link is taken as fixed, which is why the formula uses (n −1).

## B. Effect of Lower Pair

A lowerpair joint such as a revolute or prismatic joint removes two degrees of freedom from a planar system.

This is why the term 2J is subtracted in Grubler’s formula.

## C. Effect of Higher Pair

A higher pair joint normally removes one degree of freedom in a planar mechanism. Therefore, H is subtracted once. (I did not understand this concept fully)

# VIII. EXAMPLE OF DOF CALC

In a four-bar mechanism

4 links

4 revolute joints which are lower pair

0 high pair joints

Therefore,

n = 4,	J = 4,	H = 0

F = 3(n −1)−2J − H

F = 3(4−1)−2(4)−0

F = 9−8

F = 1

Therefore, the four-bar mechanism has one degree of freedom.

# IX. STEWART PLATFORM

The Stewart platform is a parallel robot that consists of a fixed base, a moving platform, and six adjustable legs.

It is also commonly called a hexapod.

The six legs connect the base to the moving platform. By changing the lengths of these legs, the position and orientation of the moving platform can be changed.

## A. Degrees of Freedom of a Stewart Platform

A Stewart platform normally has six degrees of freedom. The three translational movements are:

- Movement along x-axis

- Movement along y-axis

- Movement along z-axis

Therefore,

DOF = 3+3 = 6

The six legs work together to control these six movements.

## B. Working Principle

Each leg of the Stewart platform can change its length. The controller calculates the required length of each leg to place the moving platform in the desired position and orientation.

Since all six legs work together, the platform can make complex movements while remaining supported by the base.

Stewart platforms are used in several areas, including flight and vehicle simulators. Which can be moved in all siz Dof

# REFERENCES

—-